ScrutMan

FIA LT54 Tyre RFID Portal System

Partner Inquiry — Fixed UHF RFID Portal Reader

To: Zebra Technologies / Impinj — European Sales & Partner Team Subject: Fixed 4-port UHF RFID portal reader for motorsport scrutineering application From: ScrutMan — FIA-compliant tyre management platform

1.  Who We Are

ScrutMan is a digital scrutineering management platform built for motorsport clubs and federations operating under FIA regulations. The system handles driver registration, tyre compliance (FIA Technical List No. 54), event management, and technical inspection workflows.

We are building a physical RFID portal scanning station that integrates with our cloud platform. We are looking for the correct Zebra / Impinj reseller or partner in Europe (Norway/Scandinavia) who can supply and support fixed UHF RFID portal hardware for this application.

2.  What We Need — Summary

⚠ We do NOT need handheld scanners. ⚠ We do NOT need inventory management software (ItemSense / Zebra DNA for warehouses). ✅ We need a fixed 4-port UHF RFID reader with 4 directional antennas for a drive-through portal. ✅ We need EU/ETSI band (865–868 MHz) certified hardware. ✅ We need access to the reader SDK to integrate with our own application.

3.  Application Description

The portal is a pop-up tent installation set up at motorsport venues. A car drives slowly (5–10 km/h) through a frame fitted with 4 UHF RFID antennas. The system reads the RFID chips embedded in all 4 tyres simultaneously, validates them against the driver's registered tyre set in our platform, and triggers a green or red signal lamp within 2–3 seconds.

The FIA mandates UHF EPC Gen2 / SGTIN-96 tags (860–960 MHz) on all competition tyres per Technical List No. 54.

4.  Technical Requirements

| Requirement | Specification |
| --- | --- |
| Reader type | Fixed 4-port UHF RFID reader (e.g. Zebra FX9600-4 or Impinj R700) |
| Frequency | EU/ETSI band: 865–868 MHz (ETSI EN 302 208) |
| Protocol | EPC Class 1 Gen2 / ISO 18000-6C |
| Antenna ports | 4 ports (one per tyre / wheel position) |
| Interface | Ethernet (RJ-45) + SDK access (REST, TCP or native SDK) |
| Power | PoE or PoE+ preferred (reduces cabling at venue) |
| Environment | IP53 or better — outdoor tent, weather exposure |
| Antennas | 4× directional panel antennas, 8–12 dBi, EU-certified |
| SDK | Access to reader SDK for custom application integration (Node.js / .NET) |
| Quantity (initial) | 1 complete portal kit (reader + 4 antennas + cables) |

5.  What We Are NOT Looking For

- Handheld RFID scanners (Zebra RFD40, RFD90 or similar)

- Warehouse/logistics inventory management software (ItemSense, Zebra DNA, Impinj Platform)

- US-band only (FCC 902–928 MHz) hardware — must support EU ETSI band

- Cloud-managed enterprise RFID platforms — we have our own application

6.  Integration Approach

We have already built the server-side platform (Next.js, MySQL, REST API). The Windows desktop application (Electron + React) will connect to the reader via SDK, receive EPC strings per antenna port, validate against our API, and control a signal lamp via USB relay. We need the reader SDK documentation and ideally a demo unit or evaluation kit.

7.  Request

We would appreciate:

- Contact details for the correct authorised reseller / partner for Norway or Scandinavia

- Confirmation of product availability for EU ETSI band (865–868 MHz)

- Datasheet and SDK documentation for the recommended portal reader model

- Pricing indication for: 1× reader + 4× antennas + antenna cables

- Information about evaluation / demo kit options

Preferred models to quote: • Zebra FX9600-4 (4-port, EU/WR variant: FX9600-42325A50-WR) • Impinj R700 (EU variant, 865–868 MHz) • Or equivalent 4-port fixed UHF reader you recommend for portal/drive-through use

System Architecture Diagram

ScrutMan RFID Portal — for reference when contacting hardware partners

A.  Physical Portal Layout (top view)

┌─────────────────────────────────────────────────────────┐ │ POP-UP TENT (6 × 3 m) │ │ │ │ LEFT SIDE RIGHT SIDE │ │ ───────── ────────── │ │ [ANT 1]──┐ ┌──[ANT 2] │ │ [ANT 3]──┤ ← PORTAL FRAME → ├──[ANT 4] │ │ │ ┌──────────────┐ │ │ │ └──│ car drives │──┘ │ │ │ through → │ │ │ └──────────────┘ │ │ │ │ ANT 1+2 = ZONE 1 (Read) ANT 3+4 = ZONE 2 (Verify)│ │ │ │ 🟢 / 🔴 SIGNAL LAMP │ │ │ │ [Zebra FX9600 / Impinj R700] [Windows Mini-PC] │ │ │ └─────────────────────────────────────────────────────────┘ Antenna hub height: ~45 cm (tyre sidewall / FIA LT54 label) Car speed: 5–10 km/h through portal

B.  Antenna Port Assignment

| Antenna Port | Physical Position | Reads |
| --- | --- | --- |
| Port 1 | Zone 1 — Left side of portal | Front-Left tyre (FL) |
| Port 2 | Zone 1 — Right side of portal | Front-Right tyre (FR) |
| Port 3 | Zone 2 — Left side of portal | Rear-Left tyre (RL) |
| Port 4 | Zone 2 — Right side of portal | Rear-Right tyre (RR) |

RSSI (signal strength) from each port is used to determine wheel position. Zone 2 (ports 3+4) acts as verification — reducing missed reads to near zero.

C.  System Connection Diagram

┌───────────────────────────────────────────────────────────────────┐ │ PORTAL FRAME │ │ │ │ [Ant 1] ──┐ │ │ [Ant 2] ──┤── RP-SMA/N-type cable ──► [Zebra FX9600 / R700] │ │ [Ant 3] ──┤ 4-port UHF Reader │ │ [Ant 4] ──┘ EU: 865–868 MHz │ │ EPC Gen2 / ISO 18000-6C│ │ │ │ │ Cat6 Ethernet │ │ │ │ │ ┌──────────▼──────────┐ │ │ │ Gigabit Switch │ │ │ └──────┬───────┬───────┘ │ │ │ │ │ │ Cat6 │ │ Cat6 │ │ ┌──────────▼────┐ │ ┌──▼──────────┐ │ │ │ Windows PC │ │ │ 4G/5G Router│ │ │ │ Mini-PC │ │ │ or Starlink │ │ │ │ │ │ └──────┬──────┘ │ │ │ ScrutMan │ │ │ Internet│ │ │ Portal App │ │ ┌──────▼──────┐ │ │ │ (Electron) │ │ │ ScrutMan │ │ │ └──────┬─────────┘ │ │ Cloud API │ │ │ │ USB │ │ (Next.js) │ │ │ ┌──────▼─────────┐ │ └─────────────┘ │ │ │ USB Relay Box │ │ │ │ │ (4-channel) │ │ │ │ └──────┬─────────┘ │ │ │ │ 230V relay │ │ │ ┌────────────┴────────────┐ │ │ │ │ 🟢 / 🟡 / 🔴 │ │ │ │ │ LED Signal Lamp (×2) │ │ │ │ │ IP65, visible 30+ m │ │ │ │ └──────────────────────────┘ │ │ │ │ │ └───────────────────────────────────────────────────────────────────────┘

D.  Software Flow (for SDK integration reference)

Operator enters start number │ ▼ System arms → Amber lamp ON │ ▼ Car drives through portal (2–4 seconds) │ ▼ Reader SDK callback per tag read: { epc: 'E280117000000100…', rssi: -52.4, antenna: 1 } │ ▼ App deduplicates EPC reads within 2-second window │ ▼ For each wheel (FL / FR / RL / RR): EPC in driver's registered tyre list? └─ YES + status ACTIVE → GREEN └─ NO / unknown EPC → RED │ ▼ All 4 GREEN → Green lamp ON → Car passes Any RED → Red lamp ON → Marshal stops car │ ▼ POST /api/scan-sessions → logged in ScrutMan cloud Incident PDF generated if FAIL

E.  Approved RFID Chip Types (FIA Technical List No. 54)

The tyre RFID labels already in the field contain chips from this approved list:

| Manufacturer | Approved Models |
| --- | --- |
| Impinj | Monza 4, Monza 750, Monza R6P, M730 |
| Alien Technology | Higgs-3, Higgs-9 |
| NXP Semiconductors | UCODE 7xm-1K, UCODE 8, UCODE 9xe |

All above are EPC Gen2 / SGTIN-96 compatible — readable by any EU-certified UHF reader operating on 865–868 MHz.